CSE651                                       Emerging Applications and Platforms                              Summer 2014

Anduino Hnadout#1

- Make Arduino IDE is installed and you an Arduino board and connector ready.

- Connect the Arduino board. When it completes installing the driver make sure you note down the com port it is configured/connected to. (E.g. COM20)

- Open the Arduino Ide and study the various command menus.

- The menu items are Verify (check code for errors),  Upload (upload a program into the Arduino unit),  New ( open  a new blank program), Open (existing programs), Save (the current program).

- The Arduino IDE is written in a development language called Processing. ( We will learn about Processing later). A program written in this environment is called a “Sketch”. Processing is indeed written on top of Java as a Java library.

- Connect the Arduino board.

- Enter the following “sketch” with three parts: data definitions, setup() function and a loop() function.

/*

Blink

Turns on an LED on for one second, then off for one second, repeatedly.

This example code is in the public domain.

*/

// Pin 13 has an LED connected on most Arduino boards.

// give it a name:

//DATA AREA

int led = 13;

// the setup routine runs once when you press reset:

//SETUP

void setup() {

// initialize the digital pin as an output.

pinMode(led, OUTPUT);

}

// the loop routine runs over and over again forever:

//LOOP forever or until terminated by the user

void loop() {

digitalWrite(led, HIGH);   // turn the LED on (HIGH is the voltage level)

delay(100);               // wait for a second

digitalWrite(led, LOW);    // turn the LED off by making the voltage LOW

delay(100);               // wait for a second

}

- The data definition part defines the data  needed by the application.

- The setup() function establishes the initial setup and configuration of the devices and other variables.

- Loop keeps looping the operation specified in the function.

- Lets write a simple blink and under the basic setup.